USING DESIGN MASTER™ for

WORST CASE STABILITY ANALYSIS (BODE PLOTS)

Ed Walker

(C) 2001 Design/Analysis Consultants, Inc.

June 2001

INTRODUCTION

Design Master can be used to determine the stability phase margin limits for arbitrary first- and second-order systems described by traditional equations containing complex variables. This includes the wide variety of power supply switching regulator types, including continuous or discontinuous current with various control modes. A procedure for easily generating gain and phase formulas from the original s equations is provided in Appendices A and B. An additional procedure for generating formulas to explicitly determine phase margin is provided in Appendix C.

Using the gain and phase formulas, Design Master can compute the effect on phase margin due to the range of all component and operating point variations. If the phase margin limits do not exceed the minimum required phase margin (typically +45 degrees), then the design is OK and no further analysis is required.

If the phase margin limits do exceed the specification, then a risk assessment probability analysis can be performed. Dynamic variables (parameters which can vary during operation) are set to their worst case values, as determined from the Design Master’s table of normalized sensitivities. All other static variables (initial tolerances) are left unchanged. Using the adjusted variables, Design Master can then compute the estimated probability that the phase margin will not meet specification.

Design Master’s table of normalized sensitivities can also be used to identify the component values and operating conditions that should be applied for the most rigorous testing.

A PRACTICAL EXAMPLE

Power supply stability analysis is an essential design validation task. To demonstrate an improved general method, a sample phase margin stability analysis is performed for the popular discontinuous-mode flyback regulator. A circuit described by Abraham Pressman in Section 12.18 of Switching Power Supply Design, 2nd Edition, McGraw-Hill, is used as a reference, but with one key difference: it is assumed that the compensation amplifier was not designed as expertly as done by Mr. Pressman, but has one capacitor value at a different value. This will allow the example to not only demonstrate the analysis methodology, but to also show how a circuit may appear to be pretty good in the lab, while capable of exhibiting significant problems if produced in volume.

The basic equations and values are provided in the following two Design Master files, available at DACI’s web site: http://www.daci-wca.com/daci_014.htm

1. Bode DM Flyback Limits.dm

2. Bode DM Flyback Probabilities.dm

The first analysis (Limits) examines the possible range of phase margin, taking the variations of all variables into account. The second analysis (Probabilities) is a risk assessment. It is very similar to the first analysis, except that dynamic variable values are assigned to their worst case values. This process will be described later.

Limitations

As in Pressman’s example, higher-frequency effects such as switching time delay are considered negligible within the bandwidth of interest, although of course in a complete analysis such assumptions should be confirmed. Also, aging/time tolerances can be applied to a third-pass analysis to evaluate performance as the circuit ages.

DESIGN EQUATIONS

When working with complex variables (s equations), it is necessary to split out the gain and phase information into separate formulas. This is easily done by using the procedures provided in Appendices A and B. Although the procedures may at first look complicated, they really aren’t. With a little practice it is possible by inspection to directly convert s equations into the equivalent gain and phase formulas.

Using the procedures, Pressman’s converted design equations were entered into the worksheet, as shown below:

C2’s design value per Pressman is 2000 pf.

Note that the inverting gain of the error amplifier was accounted for by adding 180 degrees of phase shift (Pi).

VARIABLES

Next, all the circuit variables were entered in the Variables Library. Practical min/max values were assigned to all variables that were not assigned a range by Pressman.

BODE PLOTS

After entering the formulas and variables, gain and phase Bode plots were generated to show the range of performance. As can be seen, the range of variables generates a fairly broad min/max spread in both the gain and phase plots:

To find the phase margin first look at the magnitude plot Aloop to find the frequency f1 where the magnitude equals unity (0 dB). Then look at the phase plot Ploop to find the phase at f1. The phase value at f1 is the phase margin, which typically should be a positive value of at least 45 degrees for good stability.

Now if you try looking at the graphs above, it becomes apparent that the range of curves makes it difficult to identify the phase curve which corresponds to a given gain curve. In this particular example the most negative dip in the set of phase curves is not close to the actual worst case phase margin. To determine the actual phase margin, one solution would be to adjust only one variable at a time, and obtain a set of single gain and phase curves for each case. But it would become rapidly apparent that this is impractical because of the very high number of curve sets that would be required to account for all the component and operating variations. Therefore a direct means of determining phase margin is desired.

HOW TO EXPLICITLY FIND THE PHASE MARGIN

A method is described in Appendix C that directly determines the phase margin. The method is a designer-friendly process of simple algebra, which takes advantage of Design Master’s ability to solve transcendental equations (circular references).

FIRST PASS PHASE MARGIN LIMITS

The flyback circuit’s unity gain crossover and phase margin formulas were derived as explained in Appendix C, and entered into the Worksheet as shown below:

Design Master was next used to iteratively solve the equations and it was found that the phase margin could range from 32 to 81 degrees, with a typical of about 52 degrees. Therefore assuming a specification of at least +45 degrees margin, the typical lab test would easily pass. Should this design be approved?

THE PROPER USE OF VARIABLES

IN PROBABILITY ANALYSIS

Design Master can calculate both the worst case limits of performance, and the estimated chance of exceeding specification limits. In the first limits analysis, no particular regard need be applied to the nature of the variables used in the analysis; the min/max results will be accurate. If these results do not exceed specification limits, then the design is OK and the analysis is complete. However, if the min and/or max results exceed the specification limits, then the probability results may not be accurate, depending on the nature of the variables used in the analysis. To avoid overly optimistic estimates of the potential deficiency, the range of static variables (initial tolerances) should be used in the probability calculations. Dynamic variables (parameters which can change during circuit operation, e.g. temperature, load) should be set to their worst case values, as identified by Design Master’s table of normalized sensitivities. (For more information, see The Proper Use of Variables in Generating Probabilities in section 4.3, “Probability Distributions,” of the Design Master User’s Manual.)

NOTE: Design Master V5.1 automatically generates the worst case equations needed for the probability analysis.

Perhaps not, since the first-pass results also indicated a probability that the phase margin would be less than 45% about 10% of the time. But this needed to be refined, with more attention given to the types of variables used in the analysis (see box). Therefore a second probability analysis was performed.

SECOND PASS PHASE MARGIN RISK ASSESSMENT

The first-pass analysis was repeated, after first assigning worst case values to all dynamic application variables, as determined by a review of the normalized sensitivities (see next page). The second analysis determined that the circuit could have phase margin less than its +45 degrees requirement approximately 43% of the time. This won’t be seen initially for most units “out of the box”; the error rate applies when the units are exposed to the worst case application variables. When that happens the warranty repair technicians will not be happy, plus it’s time to brush up the old resume.

AN ADDED BENEFIT: GUIDANCE FOR RIGOROUS TESTING

The normalized sensitivities also indicate which variables have most effect on stability. As can be seen from the sensitivities table and tornado plot on the next page, output load resistance Ro has the most effect; its negative slope indicates that phase margin will be smallest for maximum resistance (minimum load). The next most significant effect is the tolerance for the output capacitor’s ESR; as it decreases phase margin will decrease; etc. Operating conditions are the easiest to adjust for testing. However it may be worthwhile to insert a “worst case” filter capacitor for a stability check.

TESTING THE CIRCUIT FOR MINIMUM PHASE MARGIN

Worst Case Operating Conditions (1)

Output load resistance			MAXIMUM

Input Voltage				MINIMUM

Temperature				ROOM (not sensitive)

Worst Case Component Values (2)

Output capacitor’s ESR			MINIMUM

Output capacitor			MAXIMUM

Capacitor C2				MAXIMUM

Capacitor C1				MINIMUM

Note 1: Dynamic variables are set to their worst case values for the probability analysis.

Note 2: Static variables are allowed to have their full range of values for the probability analysis.

Ro

-30.3526 %

TiniRc

27.18723 %

Vin

9.55962 %

TiniCfilt

-8.78611 %

TiniC2

-5.71212 %

TiniC1

2.94936 %

Vs

-2.12454 %

Eff

1.93368 %

L

-1.72762 %

fs

-1.72738 %

TiniR1

-1.24147 %

TtcCfilt

-1.16942 %

TiniR2

-1.12591 %

TtcR2

-1.05858 %

TtcC2

-0.99356 %

TtcR1

-0.96603 %

TtcC1

0.962 %

Ta

-0.42276 %

SENSITIVITIES

SOME FINAL NOTES

1. Pressman’s example, with the proper value of 2000 pf used for C2, has approximately 68 degrees of phase margin under nominal conditions, and a minimum phase margin of 47 degrees.

2. The example used has a fairly large error. In more typical cases the error will be much smaller and unlikely to be detected during prototype testing. The problem is that even extensive and rigorous lab testing generally only varies the dynamic application variables, not the static component tolerances. Since even small errors can ruin your profit margin or your reputation, it’s a good idea to complement rigorous testing with a thorough analysis as described herein. As always, test and analysis results should be compared to each other to crosscheck each validation method.

-----------------------------------------------------------------------------------------------------------------------------------------------

APPENDIX A

Procedure for Deriving Gain Formula from S Equation

The Bode magnitude plot of a G(s) function is obtained by arranging each term of the function into its real and imaginary parts, taking the square root of the sum of the squares, then converting to dB:

20*LOG(SQR(real^2+imaginary^2))

If more than one term, terms are multiplied/divided as in the original G(s) formula, then converted to log.

1. Conversion of terms of the form s

Gain = SQR(0 + w^2) = w			 	       [for 1/s, Gain = 1/w]

In dB, 20*LOG(w)

2. Conversion of first-order terms of the form (1 + s*a)

Gain = SQR(1 + (w*a)^2)

In dB, 20*LOG(SQR(1 + (w*a)^2))

If the terms are not in the form (1 + s*a), just rearrange the formula.

Example:  Original = (10+s)*(3+s) = 10*3(1+s/10)*(1+s/3)            [a=1/10 and 1/3]

Conversion = 10*3*SQR(1+(w/10)^2)*SQR(1+(w/3)^2)

In dB, 20*LOG(10*3*SQR(1 + (s/10)^2)*SQR(1 + (s/3)^2))

3. Conversion of second-order terms of the form (1 + s*a + s^2*b)

Gain = SQR((1-w^2*b)^2 + (w*a)^2)

In dB, 20*LOG(SQR((1-w^2*b)^2 + (w*a)^2))

Example:  Original = A/(1 + s/(wo*Q) + (s/wo)^2)  	  [a=1/(wo*Q), b=1/wo^2]

Conversion = A/SQR((1-(w/wo)^2)^2 + (w/(wo*Q))^2)

In dB, 20*LOG(A/SQR((1-(w/wo)^2)^2 + (w/(wo*Q))^2))

APPENDIX B

Procedure for Deriving Phase Formula from S Equation

The Bode phase plot of a G(s) function is obtained by taking the 4-quadrant inverse tangent of the real and imaginary parts of each term, then multiplying by 180/Pi to convert radians to degrees:

ATAN2(real, imaginary)*180/Pi

Any gain terms outside the s terms are dropped. If more than one term, add multiplied terms and subtract divided terms.

1. Conversion of terms of the form s

Phase = ATAN2(0,w) radians       	   [for 1/s, Phase = -ATAN2(0,w) radians]

In degrees, ATAN2(0,w)*180/Pi

2. Conversion of first-order terms of the form (1 + s*a)

Phase = ATAN2(1,w*a) radians

In degrees, ATAN2(1,w*a)*180/Pi

Example:  Original = (10+s)*(3+s) = 10*3(1+s/10)*(1+s/3)             [a=1/10 and 1/3]

Conversion = ATAN2(1,w/10) + ATAN2(1,w/3)

In degrees, (ATAN2(1,w/10) + ATAN2(1,w/3))*180/Pi

3. Conversion of second-order terms of the form (1 + s*a + s^2*b)

Phase = ATAN2(1-w^2*b,w*a) radians

In degrees, ATAN2(1-w^2*b,w*a)*180/Pi

Example:  Original = A/(1 + s/(wo*Q) + (s/wo)^2)  	  [a=1/(wo*Q), b=1/wo^2]

Conversion = -ATAN2(1-(w/wo)^2,w/(wo*Q))   [drop gain A and use (-) since dividing]

In degrees, (-ATAN2(1-(w/wo)^2,w/(wo*Q)))*180/Pi

APPENDIX C

Procedure for Deriving the Explicit Phase Margin Formula

1.  Start with the magnitude formula before the dB conversion

Example: Magnitude = SQR(1+(w*C1*R2)^2)/((w*C1*R1)*SQR(1+(w*C2*R2)^2))

2.  Set magnitude to 1

Example: 1 = SQR(1+(w*C1*R2)^2)/((w*C1*R1)*SQR(1+(w*C2*R2)^2))

3.  Rearrange the formula so that any single w term is on the left of the equation

Example: w*C1*R1 = SQR(1+(w*C1*R2)^2)/(SQR(1+(w*C2*R2)^2))

4. 	Rearrange the formula so that the w from the left term is alone on the left side of the equation.  This will still leave w on the right of the equation, but this is OK since Design Master can usually solve iterative equations

Example: w = SQR(1+(w*C1*R2)^2)/(SQR(1+(w*C2*R2)^2))/(C1*R1)

5. 	Convert w to 2*Pi*f1

Example: 2*Pi*f1 = SQR(1+(2*Pi*f1*C1*R2)^2)/(SQR(1+(2*Pi*f1*C2*R2)^2))/(C1*R1)

6. 	Rearrange to obtain f1

Example: f1 = SQR(1+(2*Pi*f1*C1*R2)^2)/(SQR(1+(2*Pi*f1*C2*R2)^2))/(C1*R1)/(2*Pi)

7.	Now you can solve directly for f1.  When you enter the formula for f1, Design Master will ask you for its estimated value since f1 is a circular reference.  Enter a value that is in the vicinity of unity gain (0 dB) from the original magnitude plot. If Design Master can't solve f1, you may need to use a different w term on the left of the formula (step 3).

8. 	The phase margin can be obtained directly by substituting 2*Pi*f1 for w into the original phase formula

Example: Phase Margin = 180 + (ATAN2(1,2*Pi*f1*C1*R2) -  (ATAN2(0,2*Pi*f1*C1*R1) + ATAN2(1,2*Pi*f1*C2*R2)))*180/Pi